COMPLIANCES							      B – 19

INSPECTION: 3 MONTHS

| NOTE: |  |
| --- | --- |
|  | To be used by facilities in New Zealand ONLY |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

This Job Card may be carried out by a Competent/Trained person, which verifies required and scheduled Compliance Inspections have taken place.

These checks and inspections are directly linked to the building’s “Compliance Schedule.” Routine checks are required to ensure that the building can be legally occupied and that an annual Building Warrant of Fitness (BWOF) may be obtained.

Perform the following:

- Check to see that all scheduled inspections, including Independent Qualified Person (IQP) inspections, have been made

- Ensure specified systems have been checked to the required frequency and procedure

- Ensure date, issues and/or comments have been recorded in Compliance Logbook

- Report any inspections not completed to person of Oversight

- Ensure that IQP’s are instructed to perform the annual inspections and issue the BWOF.

The following is an outline of the required inspections and checks:

(SS9/1) MECHANICAL VENTILATION & (SS9/2) AIR CONDITIONING SYSTEMS

- Mechanical Ventilation and Air Conditioning Systems shall be inspected every three months (as set out in the task instructions) by a trained person and annually inspected by an IQP

- All tests shall be recorded on the SS9 forms contained in the Compliance Log Book.

(SS14/2) SIGNS IN RELATION TO ANY SPECIFIED SYSTEMS 1-13

- Signs shall be inspected at three monthly intervals by a trained person and annually by an IQP, to ensure they are the correct type, legible and in the correct location

- All tests shall be recorded on the SS14/2 form contained in the Compliance Log Book.

(Continued)

INSTRUCTIONS

(SS15/4) SIGNS FOR COMMUNICATING INFORMATION INTENDED TO FACILITATE EVACUATION

- Signs shall be inspected at three monthly intervals by a trained person and annually by an IQP, to ensure they are the correct type, legible and in the correct location

- All tests shall be recorded on the SS15/4 form contained in the Compliance Logbook.

Needed Repairs or Additional Comments