IRRIGATION SYSTEMS	            	      		        C – 3

INSPECTION & SERVICE: 3 MONTHS

| NOTE: |  |
| --- | --- |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

Perform the following general maintenance tasks:

- Inspect fixings for signs of rust and corrosion

- Inspect tanks for signs of leaks

- Check hoses and clamps for damage or kinks

- Operate all valves

- Check overall condition of system using the controller manual operation mode

- Clear vegetation inhibiting effective coverage of sprinkler heads

- Check operation of hoses and sprinkler heads. If there is a lesser spray distance compared to others in the same zone this usually indicates a clogged or restricted screen in the head

- Check operation and programming of automatic controller (if fitted)

IRRIGATION PUMP & FILTER

- Manually operate the low water level shut-off device and check pump stops

- Isolate and lock out supplies (water and power)

- Open enclosures and clean filters. Replace as required or specified by manufacturer

- Check seals and O-rings. Replace as required or as specified by manufacturer

- Operate pump and system after filter clean to ensure there are no leaks and the pump does not have any unusual noise

Needed Repairs or Additional Comments