REVERSE CYCLE SYSTEMS       			       M – 1

INSPECTION & SERVICE: 12 MONTHS

| NOTE: |  |
| --- | --- |
|  | JHA Required Manual (M-1) |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

The following tasks apply to systems such ducted split, ceiling cassette, wall splits and package units.

OUTDOOR UNIT

- Remove leaves, plants, dirt and yard debris from around unit.

- Inspect unit for rust. Touch up as required

- Wash outside cover, clean unit surface

INDOOR UNIT (WALL SPLITS & CEILING CASSETTES)

    - Open cover and remove filters

    - Vacuum any dust or wash the air filter with natural detergent thinned with lukewarm water

    - Allow the filter to dry completely outside in a shaded area

    - Reinstate filters and covers

INDOOR UNIT (DUCTED SPLITS)

Each system should consist of outside (fresh) & return air filters. Some systems will have re-useable filters while others with have disposable filters.

- Dust inside filter covers

    - Disposable Filters (If fitted) – Replace every 12 months. Mark new filters with installation date.

    - Ensure thermostat is not obstructed or covered to allow accurate temperature monitoring

Reinstate supply and test system for correct overall operation.

Needed Repairs or Additional Comments