AUDIO SYSTEM                                               T – 1

INSPECTION & SERVICE: 12 MONTHS

| NOTE: |  |
| --- | --- |
|  | Manual (T-2) |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

Perform the following general tasks on all sound equipment. Ensure power supply for sound equipment is off and unplugged prior to work. To ensure the system will operate correctly following the service, please take a photo of the equipment settings and cable connections.

SOUND EQUIPMENT

- Using an anti-static microfiber cloth, gently clean outside of equipment

- Clean microphone, windsock and battery contacts

- Ensure microphone cables are fitted correctly and any batteries are replaced

- Use eraser or similar to clean battery contacts on wireless microphones

MICROPHONE STANDS

- Check all the functions of the stand and repair as needed

PERFORMANCE TEST

- With system on, verify all speakers, loudspeaker system and microphones are working

- With sound on, listen for crackling or distortion noises from microphones and loudspeaker system (this may occur when a connection or cable is loose)

- Check all components and controls on mixer unit function correctly. Reinstate settings as per photo taken prior to work

- Ensure all cables and wires are neatly arranged and in good condition (if adjustment is required ensure the cable stays properly connected at each end)

Needed Repairs or Additional Comments