COMPLIANCES		                       	      		B – 20

INSPECTION: 12 MONTHS

| NOTE: |  |
| --- | --- |
|  | To be used by facilities in New Zealand ONLY |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

This Job Card may be carried out by a Competent/Trained person, which verifies required and scheduled Compliance Inspections have taken place.

These checks and inspections are directly linked to the building’s “Compliance Schedule.” Routine checks are required to ensure that the building can be legally occupied and that an annual Building Warrant of Fitness (BWOF) may be obtained.

Perform the following:

- Check to see that all scheduled inspections, including Independent Qualified Person (IQP) inspections, have been made.

- Ensure specified systems have been checked to the required frequency and procedure

- Ensure date, issues and/or comments have been recorded in Compliance Logbook.

- Report any inspections not completed to person of Oversight

- Ensure that IQP’s are instructed to perform the annual inspections and issue the BWOF

The following is an outline of the required inspections and checks:

(SS2) AUTOMATIC OR MANUAL EMERGENCY WARNING SYSTEMS

- The fire alarm system shall be inspected and tested annually to NZS 4512 by an IQP

- All tests shall be recorded on the SS2 form contained in the Compliance Log Book.

(SS4) ILUMINATED EXIT LIGHTS / EMERGENCY LIGHTING SYSTEMS

- Emergency lighting systems shall be inspected annually by an IQP

- All tests shall be recorded on the SS4 form, contained in the Compliance Log Book.

(Continued)

INSTRUCTIONS

(SS15/2) FINAL EXITS

- Final exits shall be inspected annually by an IQP.  Inspections shall be made to ensure that final exits perform as designed for safe evacuation, and in particular that:

- Final exits are kept clear of obstacles and hazards

- Final exit doors are not locked, barred, or blocked so as to prevent occupants from leaving the building in the event of an emergency without the use of a key

- All tests shall be recorded on the SS15/2 form contained in the Compliance Log Book.

(SS7) AUTOMATIC BACK-FLOW PREVENTERS

- Annual inspection shall be undertaken by an IQP and recorded on the SS7 form contained in the Compliance Log Book.

(SS9/1) MECHANICAL VENTILATION & (SS9/2) AIR CONDITIONING SYSTEMS

- Mechanical Ventilation and Air Conditioning Systems shall be inspected annually by an IQP

- All tests shall be recorded on the SS9/1 and SS9/2 forms contained in the Compliance Log Book.

(SS14/2) SIGNS IN RELATION TO ANY SPECIFIED SYSTEMS 1 – 13

- Signs shall be inspected annually by an IQP, to ensure they are the correct type, legible and in the correct location

- All tests shall be recorded on the SS14/2 form contained in the Compliance Log Book.

(SS15/4) SIGNS COMMUNICATING INFORMATION INTENDED TO FACILITATE EVACUATION

- Signs shall be inspected annually by an IQP, to ensure they are the correct type, legible and in the correct location

- All tests shall be recorded on the SS15/4 form contained in the Compliance Logbook.

(BWOF) BUILDING WARRANT OF FITNESS

- Ensure that IQP’s are instructed to perform the annual inspections and issue the BWOF

- Ensure that new BWOF is displayed visibly within the building.

Needed Repairs or Additional Comments