STRUCTURAL INSPECTION		                     B – 9

INSPECTION: 12 MONTHS

| NOTE: |  |
| --- | --- |
|  | JHA Required Manual (B-1) |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

ATTIC/ROOF SPACE

- Inspect for any signs of separation or bowing of trusses and rafters

- Inspect accessible gang nail plates for signs of peeling

- Check general condition of roof structure and condition of members

- Inspect insulation. Reposition disrupted insulation batts to correct locations

BUILDING INTERIOR

- Check for misalignment of doors, windows, floors. Inspect general condition of walls for cracking

- Check for signs of moisture build-up on surfaces or walls

- Check bottom chord of trusses. Lift insulation if required

- Inspect any accessible structural members for rotting or significant splitting

- For “Tilt Up” concrete panel walls inspect exposed steelwork and embedded weld plates for corrosion and movement.

BUILDING EXTERIOR

- Check for new or widening cracks in brick, block, beams or columns. Take note of position and size so as to monitor deterioration (Taking photos may prove to be helpful)

- Check for signs of rust on concrete surfaces, which may indicate corrosion of reinforcement

Needed Repairs or Additional Comments