INTERIOR LIGHTING                                     EL – 8

INSPECTION & SERVICE: 6 MONTHS

| NOTE: |  |
| --- | --- |
|  | HIGH RISK JHA Required Qualified Person |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

Inspect all interior lights at facility for correct function. Note any lights that do not function correctly.

FLUORESCENT LIGHTS

- Remove reflector/diffuser taking care not to twist or damage

- Remove surface dust on the fixture with cloth or duster

- Change any burnt-out lamps and associated starters

- Note and report any defective parts

- Refit reflector/diffuser

LED PANELS

- Wipe the border with a soft damp cloth

- Clean the light emitting surface with a duster or micro fibre cloth.  If required, wipe the surface with a soft damp cloth using a mild detergent

- Each year check that the power cable is correctly plugged in and that the transformer is not covered with insulation

OTHER FITTINGS

- Remove and replace any burnt out bulbs, like for like

- Clean the light and reflectors with duster or cloth

- Refit covers and check light secure

After servicing all lights, turn all lights on and inspect for correct operation.

Needed Repairs or Additional Comments