SECURITY SYSTEM                                          T – 5

INSPECTION & SERVICE: 12 MONTHS

| NOTE: |  |
| --- | --- |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

Notify Alarm Company (if monitored) or local contact before performing the following:

SECURITY SYSTEM

- Check condition of security panel and control enclosure. Inspect for exposed wires or blown fuses

- Tighten any loose wires or components

- Test voltage of back up battery. Replace if output is low

- Activate each sensor (door, glass and motion)

- Verify each sensor is clean and secure. Check motion sensor indicating lights operate

- Perform a functional test, by arming system, and activating a motion sensor

SURVEILLANCE CAMERAS & HOUSINGS

- Clean exterior lenses and camera body using microfiber cloth

- Ensure that cameras and support brackets are securely mounted

- Inspect camera and junction box connections. Replace or repair as required

- Ensure components are adequately protected from water ingress

- Check clarity of images sent to monitoring system

- Check quality of playback and retrieved images stored. Adjust angles of cameras if required

Needed Repairs or Additional Comments