FLOOR COVERINGS		             	             B – 11

INSPECTION: 3 MONTHS

| NOTE: |  |
| --- | --- |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

CARPET

- Inspect carpet for any tears, lifting, bubbling or significant wear (especially in high traffic areas)

- Trim edges of carpet tiles using a sharp knife to keep edges flat

- Replace any stained or damaged carpet tiles as required

- Broadloom carpet loose threads should be glued down with clear adhesive

TILES

- Check for chipped, cracked, or loose tiles

- Check for missing, damaged, permanently wet, or mouldy grout.  Check for cracks in the tile grout

- Check the condition of the silicone joints and skirting tiles

- Check sealant at base of any floor mounted cabinetry and sanitary fixtures

- If a floor sealer has been used on the tiles, check its condition

CONCRETE

- If the floor is painted, check the paint for lifting, peeling, flaking, or wear

- Observe the general condition of the floor.  If cracks exist, make a sketch or take a photo indicating the location and approximate size. On subsequent inspections, check that the crack is not enlarging.  If it is, immediately notify the Local Design/Construction department.

Needed Repairs or Additional Comments