GATES		                         			               C – 5

INSPECTION & SERVICE: 12 MONTHS

| NOTE: |  |
| --- | --- |

Assigned:

Date Assigned:

Date Completed:

INSTRUCTIONS

MANUAL

- Check condition and operation of hinges and latches. Tighten fasteners and replace rusted hinges

- Check lock/locking mechanism functioning properly. Lubricate with a silicone-based aerosol spray

- Lubricate the hinges with grease gun

- Check the gate for correct fit and proper closing

- Check paint finish

- Clean any tracks and inspect wheels for operation (if fitted)

ELECTRIC

- Check that the electrical connections are in good order

- Check that the electrical box is waterproof

- Check correct operation of that reflectors, loop detector, limit switches, remote controls

- Check that the gate opens smoothly without catching and it runs to the end of its travel

Needed Repairs or Additional Comments